ADITYA INSTITUTE OF TECHNOLOGY & MANAGEMENT, TEKKALI

AUTONOMOUS

DEPARTMENT OF MECHANICAL ENGINEERING

Class: III MECH (I Sem)			MID-I		         		Date: 02-08-2017

Subject: DOM				2017-18		        		Time: 2 Hours

Answer all the following questions.				10x4 = 40M

|  | Course Outcome (s) |
| --- | --- |
| a) When the crank is at the inner dead centre, in a reciprocating steam engine, then the velocity of the piston will be _____ [ ] (A) Minimum (B) Zero (C) Maximum (D) None of these | CO 1 |
| b) Maximum fluctuation of energy in a flywheel is equal to _____ [ ] (where I = Mass moment of inertia of the flywheel, E = Maximum fluctuation of energy, CS = Coefficient of fluctuation of speed, and ω = Mean angular speed) (A) Iω(ω1 -ω2) (B) Iω2CS (C) 2 ECS (D) all of these | CO 1 |
| c) Maximum fluctuation of energy is the [ ] (A) difference between the maximum and minimum energies (B) sum of the maximum and minimum energies (C) variations of energy above and below the mean resisting torque line (D) ratio of the mean resisting torque to the work done per cycle | CO 1 |
| ## d) Which of the following statement is correct? [ ] (A) Flywheel reduces speed fluctuations during a cycle for a constant load, but flywheel does not control the mean speed of the engine if the load changes (B) Flywheel does not reduce speed fluctuations during a cycle for a constant load, but flywheel does control the mean speed of the engine if the load changes. (C) Governor controls speed fluctuations during a cycle for a constant load, but governor does not control the mean speed of the engine if the load changes. (D) Governor controls speed fluctuations during a cycle for a constant load, and governor also controls the mean speed of the engine if the load changes. | CO 1 |
| ### e) If Cf is the coefficient of speed fluctuation of a flywheel then the ratio of ### ωmax / ωmin will be: _____ [ ] (A) 1-2Cf1+2Cf (B) 2-Cf2+Cf (C) 1+2Cf1-2Cf (D) 2+2Cf2-Cf | CO 1 |
| ### f) If the rotating mass of a rim type flywheel is distributed on another rim type flywheel whose mean radius is half the mean radius of the former, then energy stored in the latter at the same speed will be [ ] ### (A) four times the first one (B) same as the first one ### (C) one fourth of the first one (D) one and a half times the first one | CO 1 |
| g) When the load on the engine increases, it becomes necessary to increase the supply of working fluid and when the load decreases, less working fluid is required. The supply of the working fluid to the engine is controlled by a ____ [ ] (A) D-slide valve (B) Governor (C) Meyer's expansion valve (D) Flywheel | CO 2 |
| h) The ratio of height of Porter governor (when length of arms and links are equal) to the height of Watt's governor is (where m = Mass of the ball, and M = Mass on the sleeve) [ ] (A) (B) (C) (D) | CO 2 |
| i) A Hartnell governor is a _____ type governor. [ ] (A) pendulum (B) dead weight (C) inertia (D) spring loaded | CO 2 |
| j) When the sleeve of a Porter governor moves downwards, the governor speed _[ ] (A) increases (B) decreases (C) remains unaffected (D) first increases, then decreases | CO 2 |

Part-B

|  |  | Course Outcome (s) | Cognitive Level |
| --- | --- | --- | --- |
|  | Derive expressions for displacement, velocity, accelerations, crank & piston efforts of reciprocating parts in slider crank mechanism. [10 M] | CO 1 | Application |
|  | The turning moment diagram for a petrol engine is drawn to vertical scale of 1mm to 6N-m and a horizontal scale of 1mm to 10. The turning moment repeats itself after every half revolution of the engine. The areas above and below the mean torque line are 305, 710, 50, 350, 980 and 275 mm2. The rotating parts amount to a mass of 40kg at a radius of gyration of 140 mm. Calculate the coefficient of fluctuation of speed if the mean speed of engine is 1500 rpm. . [10 M] | CO 1 | Application |
|  | In a porter governor, each of the four arms is 400mm long. The upper arms pivoted on the axis of the sleeve, whereas the lower arms are attached to the sleeve at a distance of 45 mm from the axis of rotation. Each ball has a mass of 8kg and the load on the sleeve is 60 kg. What will be the equilibrium speeds for the two extreme radii of 250 mm and 300 mm of rotation of the governor balls? | CO 2 | Application |

CO-PO & PSO Mapping

| CO | CO1 | CO2 | CO3 | CO4 | CO5 |
| --- | --- | --- | --- | --- | --- |
| PO | 1, 2, 3, 6, 12 | 1, 2, 3, 6, 12 |  |  |  |
| PSO | 1, 2 | 1, 2 |  |  |  |