ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AUTONOMOUS)

MID – I 				Date: 2-08-2017

Branch: B. Tech, ME_A&B 	     Year / Semester: III – I (2017-18)			Time: 2 hrs

Subject: MC & MT		          	  					 		Marks: 40

ANSWER ALL QUESTIONS			  		     	    	 4 X 10 = 40 MARKS

- a) The blade of a power saw is made of 	   (CO1	)					 [	]

A) boron steel		B) high speed steel  	C) stainless steel	D) malleable cast iron

b) The function of chip breaker is 	   (CO1	)					 [	]

A) To reduce radius of curvature of chip	B) To increase radius of curvature of chip

C) To increase ductility of work piece	D) To increase toughness of chip

c) Crater wear occurs on		   (CO1	)					 	 [	]

A) Tool face	   	B) Tool flank		C) Tool nose		D) Tool shank

d) Cutting power consumption in turning can be significantly reduced by    (CO1)	 [	]

A) increasing rake angle of tool	   	B) increasing the cutting angle of the tool

C) widening the nose radius of the tool	D) increasing the clearance angle

e) The type of chip produced when cutting cast-iron is 	   (CO1	)			 [	]

A) Continuous    				B) Discontinuous

C) Built-up edge				D) Continuous with built up edge

f) If tool designation is      8 -14 -6 -6 -6 -15 -in ASA system, what is 14?   (CO1)	 [	] 	         	A) Back rake angle 	B) Side rake angle 	C) End relief angle 	D) Side relief angle

g) If tool designation is      0 -10 -6 -6 -8 -90 -1mm in ORS system, what is 10?   (CO1)[	] 	 A) Back rake angle 	B) Orthogonal rake angle 	C) End relief angle 	D) Side relief angle

h) A dynamometer is a device used for the measurement of    (CO1)	                          [         ]

(A) chip thickness ratio 			(B) forces during metal cutting

(C) wear of the cutting tool 			(D) deflection of the cutting tool

i) The most common included angle of work supporting center of lathe   (CO2	)	 [	]

A) 1000		B) 900			C) 600			D) 450

j) Knurling operation in a lathe is used for producing	   (CO2	)			  [	]

(A) a plane surface 				(B) a cylindrical surface

(C) a serrated surface				(D) a tapered surface

| Q. No. | Question | Course outcome(s) | Cognitive Level |
| --- | --- | --- | --- |
| 2. | Explain briefly about mechanisms of various chip formation in metal cutting with neat sketches and draw the Merchant’s force circle diagram with assumptions in metal cutting [10Marks] | CO1 | Comprehension |
| 3. | Explain various types of cutting tool materials with their properties. [10Marks] | CO1 | Comprehension |
| 4. | What are the various operations performed on lathe with neat sketches. [10 Marks] | CO2 | Comprehension |

| CO | POs | PSOs |
| --- | --- | --- |
| 1 | 1,10 | 3 |
| 2 | 1,5,10 | 3 |

******

ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AUTONOMOUS)

MID – I 				Date: 2-08-2017

Branch: B. Tech, ME_A&B 	     Year / Semester: III – I (2017-18)			Time: 2 hrs

Subject: MC & MT		          	  					 		Marks: 40

ANSWER ALL QUESTIONS			  		     	    	 4 X 10 = 40 MARKS

- a) The blade of a power saw is made of 	   (CO1	)					 [	]

A) boron steel		B) high speed steel  	C) stainless steel	D) malleable cast iron

b) The function of chip breaker is 	   (CO1	)					 [	]

A) To reduce radius of curvature of chip	B) To increase radius of curvature of chip

C) To increase ductility of work piece	D) To increase toughness of chip

c) Crater wear occurs on		   (CO1	)					 	 [	]

A) Tool face	   	B) Tool flank		C) Tool nose		D) Tool shank

d) Cutting power consumption in turning can be significantly reduced by    (CO1)	 [	]

A) increasing rake angle of tool	   	B) increasing the cutting angle of the tool

C) widening the nose radius of the tool	D) increasing the clearance angle

e) The type of chip produced when cutting cast-iron is 	   (CO1	)			 [	]

A) Continuous    				B) Discontinuous

C) Built-up edge				D) Continuous with built up edge

f) If tool designation is      8 -14 -6 -6 -6 -15 -in ASA system, what is 14?   (CO1)	 [	] 	         	A) Back rake angle 	B) Side rake angle 	C) End relief angle 	D) Side relief angle

g) If tool designation is      0 -10 -6 -6 -8 -90 -1mm in ORS system, what is 10?   (CO1)[	] 	 A) Back rake angle 	B) Orthogonal rake angle 	C) End relief angle 	D) Side relief angle

h) A dynamometer is a device used for the measurement of    (CO1)	                          [         ]

(A) chip thickness ratio 			(B) forces during metal cutting

(C) wear of the cutting tool 			(D) deflection of the cutting tool

i) The most common included angle of work supporting center of lathe   (CO2	)	 [	]

A) 1000		B) 900			C) 600			D) 450

j) Knurling operation in a lathe is used for producing	   (CO2	)			  [	]

(A) a plane surface 				(B) a cylindrical surface

(C) a serrated surface				(D) a tapered surface

| Q. No. | Question | Course outcome(s) | Cognitive Level |
| --- | --- | --- | --- |
| 2. | Explain briefly about mechanisms of various chip formation in metal cutting with neat sketches and draw the Merchant’s force circle diagram with assumptions in metal cutting [10Marks] | CO1 | Comprehension |
| 3. | Explain various types of cutting tool materials with their properties. [10Marks] | CO1 | Comprehension |
| 4. | What are the various operations performed on lathe with neat sketches. [10 Marks] | CO2 | Comprehension |

| CO | POs | PSOs |
| --- | --- | --- |
| 1 | 1,10 | 3 |
| 2 | 1,5,10 | 3 |

******