ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT

(AN AUTONOMOUS INSTITUTION)

DEPARTMENT OF MECHANICAL ENGINEERING

MID EXAMINATION – I                         Date: 04-08-2017

Branch: B. Tech, ME                 Year / Semester: III –I (2017-18)		     Time: 2 hrs

Subject: TE-II								                  Marks: 40

Answer all the questions                                    Part A                                                       1×10=10 M

| Q. No. | Question | Course Outcome |
| --- | --- | --- |
| 1 | A. Rankine cycle comprises of a) Two isentropic processes and two constant volume processes b) Two isentropic processes and two constant pressure processes c) Two isothermal processes and two constant volume processes d) None | CO1 |
|  | B. Thermal efficiency of Rankine cycle is increased by - increasing the average temperature at which heat is added to the cycle - decreasing the average temperature at which heat is rejected to the cycle c) both a and b d) None of the above | CO 1 |
|  | C. The method to improve thermal efficiency of Rankine cycle a) Regenerative cycle b) Reheat cycle c) Binary vapour cycle d) All | CO 1 |
|  | D. Steam condition at inlet to turbine is super heated then work out of turbine a) Increases b) Decreases c) first increases then decreases d) not affected | CO 1 |
|  | E. Reheat cycle can improve a) boiler pressure b) work done per kg of steam used c) steam generating capacity d)none of the above | CO 1 |
|  | F. Regenerative cycle advantageous due to a) Feed water heating b) Larger work ratio c) both a and b d) none of the above | CO 1 |
|  | G. Binary vapour cycle working fluids are a) Mercury and water b) Mercury only c) Water only d) none of the above | CO 1 |
|  | H. The selection of type and size of a steam boiler depends upon a) Power required and working pressure b) Geographical position of the power house c) The fuel and water availability d) all of the above | CO 2 |
|  | I. In a boiler water passes through the tubes which are surrounded by flames and got gases called a) Fire tube boiler b) Water tube boiler c) Both a and b d) None | CO 2 |
|  | J. Which of the following is a fire tube boiler? a) Lancashire boiler b) Babcock and Wilcox boiler c)both a and b d) none of the above | CO 2 |

Part B

Answer all the questions                                                                                               3×10=30 M

| Questions |  | Course Outcome’s | PO’s &PSO’s | Cognitive level |
| --- | --- | --- | --- | --- |
| 2 | - Show Regenerative cycle and draw the T-s and h-s diagrams of Regenerative Cycle? [5 M] | CO 1 | PO’s:1,2,3,7. PSO’s: 2 | Comprehension |
|  | - Explain the working of ORSAT apparatus with neat sketch? [5 M] | CO 1 | PO’s:1,2,3,7. PSO’s: 2 | Analysis |
| 3 | - What is a boiler? How do you classify boilers? [5 M] | CO 2 | PO’s:1,2,3,4,7. PSO’s: 2 | knowledge |
|  | - Explain with a neat sketch working of Cochran boiler? [5 M] | CO 2 | PO’s:1,2,3,4,7. PSO’s: 2 | knowledge |
| 4 | A power generating plant uses steam as a working fluid and operates at a boiler pressure of 50 bar, dry saturated and a condenser pressure of 0.05 bar. Calculate cycle efficiency, work ratio and specific steam consumption for Carnot and Rankine cycle. Take pumping work for Rankine cycle? [10M] | CO 1 | PO’s:1,2,3,7 PSO’s: 2 | Application |

| Course Outcome | PO 1 | PO 2 | PO 3 | PO 4 | PO 5 | PO 6 | PO 7 | PO 8 | PO 9 | PO 10 | PO 11 | PO 12 | PSO 1 | PSO 2 | PSO 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO 1 | 3 | 2 | 1 |  |  |  | 2 |  |  |  |  |  |  | 3 |  |
| CO 2 | 3 | 2 | 2 | 2 |  |  | 2 |  |  |  |  |  |  | 3 |  |

ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT

(AN AUTONOMOUS INSTITUTION)

DEPARTMENT OF MECHANICAL ENGINEERING

MID EXAMINATION – I                         Date: 04-08-2017

Branch: B. Tech, ME                 Year / Semester: III –I (2017-18)		     Time: 2 hrs

Subject: TE-II								                  Marks: 40

Answer all the questions                                    Part A                                                       1×10=10 M

| Q. No. | Question | Course Outcome |
| --- | --- | --- |
| 1 | A. Rankine cycle comprises of a) Two isentropic processes and two constant volume processes b) Two isentropic processes and two constant pressure processes c) Two isothermal processes and two constant volume processes d) None | CO1 |
|  | B. Thermal efficiency of Rankine cycle is increased by - increasing the average temperature at which heat is added to the cycle - decreasing the average temperature at which heat is rejected to the cycle c) both a and b d) None of the above | CO 1 |
|  | C. The method to improve thermal efficiency of Rankine cycle a) Regenerative cycle b) Reheat cycle c) Binary vapour cycle d) All | CO 1 |
|  | D. Steam condition at inlet to turbine is super heated then work out of turbine a) Increases b) Decreases c) first increases then decreases d) not affected | CO 1 |
|  | E. Reheat cycle can improve a) boiler pressure b) work done per kg of steam used c) steam generating capacity d)none of the above | CO 1 |
|  | F. Regenerative cycle advantageous due to a) Feed water heating b) Larger work ratio c) both a and b d) none of the above | CO 1 |
|  | G. Binary vapour cycle working fluids are a) Mercury and water b) Mercury only c) Water only d) none of the above | CO 1 |
|  | H. The selection of type and size of a steam boiler depends upon a) Power required and working pressure b) Geographical position of the power house c) The fuel and water availability d) all of the above | CO 2 |
|  | I. In a boiler water passes through the tubes which are surrounded by flames and got gases called a) Fire tube boiler b) Water tube boiler c) Both a and b d) None | CO 2 |
|  | J. Which of the following is a fire tube boiler? a) Lancashire boiler b) Babcock and Wilcox boiler c)both a and b d) none of the above | CO 2 |

Part B

Answer all the questions                                                                                               3×10=30 M

| Questions |  | Course Outcome’s | PO’s &PSO’s | Cognitive level |
| --- | --- | --- | --- | --- |
| 2 | - Show Regenerative cycle and draw the T-s and h-s diagrams of Regenerative Cycle? [5 M] | CO 1 | PO’s:1,2,3,7. PSO’s: 2 | Comprehension |
|  | - Explain the working of ORSAT apparatus with neat sketch? [5 M] | CO 1 | PO’s:1,2,3,7. PSO’s: 2 | Analysis |
| 3 | - What is a boiler? How do you classify boilers? [5 M] | CO 2 | PO’s:1,2,3,4,7. PSO’s: 2 | knowledge |
|  | - Explain with a neat sketch working of Cochran boiler? [5 M] | CO 2 | PO’s:1,2,3,4,7. PSO’s: 2 | knowledge |
| 4 | A power generating plant uses steam as a working fluid and operates at a boiler pressure of 50 bar, dry saturated and a condenser pressure of 0.05 bar. Calculate cycle efficiency, work ratio and specific steam consumption for Carnot and Rankine cycle. Take pumping work for Rankine cycle? [10M] | CO 1 | PO’s:1,2,3,7 PSO’s: 2 | Application |

| Course Outcome | PO 1 | PO 2 | PO 3 | PO 4 | PO 5 | PO 6 | PO 7 | PO 8 | PO 9 | PO 10 | PO 11 | PO 12 | PSO 1 | PSO 2 | PSO 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO 1 | 3 | 2 | 1 |  |  |  | 2 |  |  |  |  |  |  | 3 |  |
| CO 2 | 3 | 2 | 2 | 2 |  |  | 2 |  |  |  |  |  |  | 3 |  |