ADITYA INSTITUTE OF TECHNOLOGY & MANAGEMENT, TEKKALI

AUTONOMOUS

DEPARTMENT OF MECHANICAL ENGINEERING

Class: III MECH (I Sem)			MID-II		         		Date: 30-08-2017

Subject: DOM				2017-18		        		Time: 2 Hours

Answer all the following questions.				10x4 = 40M

|  | Course Outcome (s) |
| --- | --- |
| a) The gyroscopic acceleration is given by ______ [ ] (A) δω/δt (B) ω δϴ/δt (C) r δϴ/δt (D) r δω/δt | CO 2 |
| b) The axis of spin, axis of precession and the axis of gyroscopic torque are in ___ planes. (A) two parallel (B) two perpendicular (C) three perpendicular (D) three parallel | CO 2 |
| c) If the air screw of an aeroplane rotates clockwise when viewed from rear, and the aeroplane take a right turn the gyroscopic effect will _____ [ ] (A) tend to raise the tail and depress the nose (B) tend to raise the nose and depress the tail (C) tilt the aeroplane about spin axis (D) none of the above | CO 2 |
| ## d) The effect of gyroscopic torque on the naval ship when it is rolling and the rotor is spinning about longitudinal axis is _____ [ ] (A) to raise the bow and lower the stern (B) to lower the bow and raise the stern (C) to turn the ship to one side (D) No effect | CO 2 |
| ### e) _____ friction is said to occur when there is relative motion between two completely unlubricated surfaces. [ ] (A) skin (B) greasy (C) film (D) dry | CO 3 |
| ### f) According to laws of friction, following is/are correct [ ] ### (A) friction force is directly proportion to normal reaction between two surfaces ### (B) friction force is independent of the area of contact ### (C) friction force is dependent upon the materials of two surfaces ### (D) All A, B & C | CO 3 |
| g) The efficiency of a screw jack depends on [ ] (A) pitch of threads (B) load (C) pitch and load (D) neither pitch nor load | CO 3 |
| h) For a safe design, a friction clutch is designed assuming [ ] (A) uniform pressure (B) uniform wear (C) non unifrom pressure (D) non uniform wear | CO 3 |
| i) The maximum value of angle of inclination of the plane with horizontal when body starts sliding on its own is called _____. [ ] (A) helix angle (B) inclined angle (C) angle of repose (D) coefficient of friction | CO 3 |
| j) For finding the maximum efficiency of screw, the efficiency needs to differentiated with respect to _[ ] (A) load (B) limiting angle of friction (C) lead (D) helix angle | CO 3 |

Part-B

|  |  | Course Outcome (s) | Cognitive Level |
| --- | --- | --- | --- |
|  | - What are the gyroscopic effects on a naval ship due to steering, pitching and rolling? [4 M] | CO 2 | Comprehension |
|  | - An aeroplane flying at 240 kmph turns towards the left and completes a quarter circle of 60 m radius. The mass of the rotary engine and propeller of the plane is 450kg with a radius of gyration of 320 mm. The engine speed is 2000 rpm clockwise when viewed from rear end. Determine the gyroscopic couple on aircraft and state its effect. Also determine in what way the effect changes when the aircraft turns towards its right. [6 M] | CO 2 | Analysis |
|  | - The force required just to move a body on a rough horizontal surface by pulling is 320 N inclined at 30º and by pushing is 380 N at the same angle. Find the weight of the body and coefficient of friction. [4M] | CO 3 | Application |
|  | - Derive the expression for efficiency of a square threaded screw used as a jack. Also derive the condition for maximum efficiency and find its maximum efficiency. [6 M] | CO3 | Analysis |
|  | - Write short notes on collar and pivot friction bearings. [3 M] | CO 3 | Comprehension |
|  | - Derive the expressions of frictional torque for a flat pivot bearing using (i) Uniform pressure theory and (ii) uniform wear theory. [7 M] | CO3 | Analysis |

CO-PO & PSO Mapping

| CO | CO1 | CO2 | CO3 | CO4 | CO5 |
| --- | --- | --- | --- | --- | --- |
| PO |  | 1, 2, 3, 6, 12 | 1, 2, 3, 6, 12 |  |  |
| PSO |  | 1, 2 | 1, 2 |  |  |