ADITYA INSTITUTE OF TECHNOLOGY & MANAGEMENT, TEKKALI

AUTONOMOUS

DEPARTMENT OF MECHANICAL ENGINEERING

Class: IV MECH (I Sem)			MID-I		         	             Date: 02-08-2017

Subject: FEM				            2017-18	        	             		Time: 2 Hours

Answer all the following questions.					10x4 = 40M

|  | Course Outcome(s) |
| --- | --- |
| a) Division of the given structure into discrete number of elements with degree of continuity is called [ ] (A) Discretization (B) Meshing (C) Finite element modeling (D) All a, b, &c | CO1 |
| b) The order of [D] matrix for in plane stress condition is [ ] (A)3 x 1 (B) 3 x 3 (C) 6 x 6 (D) 6 x 1 | CO1 |
| c) In a plane strain problem if σx = 20000 psi, σy = -10000 psi, E = 3 X 107 psi, and ν=0.3, What is the value of the stress σz ? [ ] (A) 3000 psi (B) zero (C) 9000 psi (D) Cannot be found | CO1 |
| d) Potential Energy of an elastic body is the sum of _________ [ ] (A) strain energy and kinetic energy (B) kinetic energy and internal energy (C) strain energy and work potential (D) kinetic energy and work potential | CO1 |
| e) The solution provided by FEM is _____ [ ] (A) Mostly approximate (B) very accurate (C) exact (D) None | CO1 |
| f) For the rod shown in figure, if εx = 2+x2, then what is the value of the tip displacement δ? [ ] (A) L (B) 2(L + L3)/3 (C) 2L + L3 (D) 2L + (L3)/3 | CO1 |
| g) For a 1D bar element with two nodes the element strain displacement matrix B is [ ] (A) 1/le (B) [-1 1] (C) 1/le [-1 1] (D) 1/le [1 -1] | CO2 |
| h) Assuming E, A, and le being unity and q1, q2 being the nodal displacements the element strain energy is (A) q1 - q2 (B) q2 - q1 (C) 1/2(q12 - q22) (D) 1/2(q1-q2)2 | CO2 |
| i) If the number of nodes is 4 for a structure modeled 1D bar with two nodes then order of global stiffness matrix is [ ] (A) 2 x 2 (B) 4 x4 (C) 3 x 3 (D) 5 x 5 | CO2 |
| j) If the number of elements is 4 for a structure modeled 1D bar with two nodes then the total number of nodes is _____ [ ] (A) 5 (B) 4 (C) 2 (D) 3 | CO2 |

Part-B

|  |  | Course Outcome (s) | Cognitive Level |
| --- | --- | --- | --- |
| 2 | a) From the generalized Hookes law of Stress-Strain relations for a 3D body, derive the material matrix D. From this derive the D matrix for plane strain condition. [7M] | CO 1 | Analysis |
|  | b) From the generalized Hookes law of Stress-Strain relations derive the material matrix D considering plane stress condition. [3M] | CO 1 | Analysis |
| 3 | a) State the six strain displacement relations for a 3D body. [4 M] | CO 1 | Application |
|  | b) Use Rayleigh Ritz method to find the displacement of the midpoint of the rod shown in Fig.1 below. [6M] | CO 1 | Application |
| 4 | Consider the bar shown below. An axial load at node 2, P = 2 X 105 N is applied. - Assemble K and F matrices. - Determine nodal displacements and stress in each element, considering that element 1 is made of Aluminum and has E1 = 7 X 1010 N/m2, A1 = 900 mm2, and element 2 is made of steel and has E2 = 2 X 1011 N/m2, A2 = 400 mm2. [10] | CO 2 | Application |

CO-PO & PSO Mapping

| CO | CO1 | CO2 | CO3 | CO4 | CO5 |
| --- | --- | --- | --- | --- | --- |
| PO | 1,2,3 | 1,2,3,5 |  |  |  |
| PSO | 1 | 1 |  |  |  |