ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AN AUTONOMOUS INSTITUTION)

MID EXAMINATION – I                         Date: 03.08.2017

Branch: MECH	                        Year / Semester: IV–I (2017-18)                                   Time: 2 hrs.

Subject: IHP                                                                                                                                Marks: 40

Answer all the questions		   PART-A   		        	                          4 X 10 = 40 Marks

| S.NO | Questions | Course outcome (s) | PO/PSO(s) | Cognitive Level |
| --- | --- | --- | --- | --- |
| 1 | i)Pressure of 1 bar equals to A) 14.5 psi B) 145 psi C) 12.5 psi D) 145x10-6 psi ii)Positive displacement pump used in hydraulic systems have A) high viscosity of fluids B) low efficiency C) required volume of fluid cannot be discharged D) all of the above iii) Which type of displacement is observed in gear pumps? A) only variable displacement B) only fixed displacement C) both fixed and variable displacement D) none of the above iv)Which energy is converted into mechanical energy by the hydraulic cylinders? A) hydrostatic energy B) hydrodynamic energy C)electrical energy D) none of the above v)Telescopic cylinders have A) only two stage unitsB) only three stage units C) two or three stage units D) multistage units vi) The rotation of gears in internal gear pump takes place in A) same directionB) different direction C) both A & B D) None vii)what is the advantage of using a single acting cylinder? A) high cost and reliable B) honing inside the inner surface of pump is not required C) piston seals are not required D) all the above viii) What does the number in 4/2 valve means? A) 4 positions and 2 ways B) 4 ways and 2 positions C) 6 positions D) none of the above ix)What is the function of a flow control valve? A) flow control valve changes the direction of oil flow B) flow control valve can adjust the flow rate of hydraulic oil C) both a & b D) none of the above x)A valve is a device that receives an external signal to release, ------ the fluid that flows through it. A) stop B) redirect C) Stop or redirect D) None of the above | CO 1 CO 1 CO 1 CO 1 CO 1 CO 1 CO 1 CO 1 CO 1 CO 2 CO 2 CO 2 | PO1,3,5,11,12 PSO 2,3 | knowledge |

PART-B

| Q. No. | Question | Course Outcomes(S) | Program Outcomes/PSO | Cognitive Level |
| --- | --- | --- | --- | --- |
| 2 | A) Define the term “Fluid power” and Explain the Desirable properties of hydraulic fluids. (5 Marks) B) Differentiate between first-class, second-class, third-class lever systems used with hydraulic cylinders to drive loads. (5 Marks) | CO 1 CO 1 | PO1,3,5,11, 12 PSO 2,3 | knowledge & Application |
| 3 | A) Explain the construction and working of a external gear pump. (5 Marks) B) A hydrostatic transmission operating at 105 bar pressure has the following characteristics: Pump; Motor. Vd= 100 cm3 ɳv= 85% ɳm = 90% N =1000 rpm; Vd = ? ɳv= 94% ɳm = 92% N =600 rpm. Find the (a) displacement of motor and (b) motor output torque. (5 Marks) | CO 1 | PO1,3,5,11, 12 PSO 2,3 | Knowledge & Appilication |
| 4 | A) How are DCVs classified? (3 Marks) B) What is the purpose of directional control valve and explain the different types of directional control valves. (7 Marks) | CO 2 | PO1,3,5,11, 12 PSO 2,3 | knowledge & Application |