ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AN AUTONOMOUS INSTITUTION)

MID EXAMINATION – I                         Date: 03.08.2017

Branch: MECH	                        Year / Semester: IV–I (2017-18)                                  Time: 2 hrs.

Subject: PPE                                                                                                                              Marks: 40

Answer all the questions                              Part-A                                                     1X10=10M

| Q. No | Question |  | Course outcomes |  |
| --- | --- | --- | --- | --- |
| 1. | A. The function of solar collector is to convert a) solar energy into electricity b) solar energy into radiation c) solar energy into thermal energy. d) solar energy into mechanical energy |  | CO 1 |  |
|  | B. The pyranometer is used for measurement of a) direct radiation only b) diffuse radiation only c) direct as well as diffuse radiation. d) none |  | CO 1 |  |
|  | C. Maximum wind energy available is proportional to a) square of the diameter of rotor b) air density c) cube of the wind velocity d) a, b and c |  | CO 1 |  |
|  | D. ……… the type of wind mill is of simple design a) Horizontal axis wind mill b) Vertical axis of the wind mill c) None. d) inclined |  | CO 1 |  |
|  | E. Fuel cell is device in which a) chemical energy is converted in to electricity b) heat energy is first converted into electricity c) heat energy is converted into electricity. d) mechanical energy into electricity |  | CO 1 |  |
|  | F. In MHD generator the conductor employed is a) gas b) liquid gas c) liquid metal or gas d) a, b and c |  | CO1 |  |
|  | G. Bio gas consists of a) only methane b) methane and CO2 with some impurities c) a special organic gas d) none of the above |  | CO 1 |  |
|  | H. Economiser is used to heat a) air b) feed water c) fuel gas d) all the above |  | CO 2 |  |
|  | I. Artificial draft is produced by a) air fan b) steam jet c) fan or stem jet d) all the above |  | CO 2 |  |
|  | J. Rankine cycle efficiency of good steam power plant may be in the range of a) 15 to 20% b) 35 to 40% c) 70 to 80% d) 90 to 95% |  | CO 2 |  |
| Part-B |  |  |  |  |
| Q. No. | Question | Course Outcomes(S) |  | Cognitive Level |
| 2. | a) List the various non- conventional energy sources [3 M] | CO 1 |  | Knowledge |
|  | b) Explain with the neat diagram a horizontal axis wind electric generating power plant [7M] | CO 1 |  | Understand |
| 3. | a) Describe open cycle MHD system [ 5M] | CO 1 |  | Knowledge |
|  | b) Explain about fuel cell ? [5M] | CO 1 |  | Understand |
| 4. | a) Give layout of modern steam power plant explain it briefly [5M] | CO 2 |  | Knowledge |
|  | b) Enumerate and explain the steps involved in handling of the coal. [5M] | CO 2 |  | Undeerstand |

| CO’s | PO | PSO |
| --- | --- | --- |
| CO 1 | 1,3,7 | 2 |
| CO 2 | 1,3,7 | 2 |

ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AN AUTONOMOUS INSTITUTION)

MID EXAMINATION – I                         Date: 03.08.2017

Branch: MECH	                        Year / Semester: IV–I (2017-18)                                  Time: 2 hrs.

Subject: PPE                                                                                                                              Marks: 40

Answer all the questions                              Part-A                                                     1X10=10M

| Q. No | Question |  | Course outcomes |  |
| --- | --- | --- | --- | --- |
| 1. | A. The function of solar collector is to convert a) solar energy into electricity b) solar energy into radiation c) solar energy into thermal energy. d) solar energy into mechanical energy |  | CO 1 |  |
|  | B. The pyranometer is used for measurement of a) direct radiation only b) diffuse radiation only c) direct as well as diffuse radiation. d) none |  | CO 1 |  |
|  | C. Maximum wind energy available is proportional to a) square of the diameter of rotor b) air density c) cube of the wind velocity d) a, b and c |  | CO 1 |  |
|  | D. ……… the type of wind mill is of simple design a) Horizontal axis wind mill b) Vertical axis of the wind mill c) None. d) inclined |  | CO 1 |  |
|  | E. Fuel cell is device in which a) chemical energy is converted in to electricity b) heat energy is first converted into electricity c) heat energy is converted into electricity. d) mechanical energy into electricity |  | CO 1 |  |
|  | F. In MHD generator the conductor employed is a) gas b) liquid gas c) liquid metal or gas d) a, b and c |  | CO1 |  |
|  | G. Bio gas consists of a) only methane b) methane and CO2 with some impurities c) a special organic gas d) none of the above |  | CO 1 |  |
|  | H. Economiser is used to heat a) air b) feed water c) fuel gas d) all the above |  | CO 2 |  |
|  | I. Artificial draft is produced by a) air fan b) steam jet c) fan or stem jet d) all the above |  | CO 2 |  |
|  | J. Rankine cycle efficiency of good steam power plant may be in the range of a) 15 to 20% b) 35 to 40% c) 70 to 80% d) 90 to 95% |  | CO 2 |  |
| Part-B |  |  |  |  |
| Q. No. | Question | Course Outcomes(S) |  | Cognitive Level |
| 2. | a) List the various non- conventional energy sources [3 M] | CO 1 |  | Knowledge |
|  | b) Explain with the neat diagram a horizontal axis wind electric generating power plant [7M] | CO 1 |  | Understand |
| 3. | a) Describe open cycle MHD system [ 5M] | CO 1 |  | Knowledge |
|  | b) Explain about fuel cell ? [5M] | CO 1 |  | Understand |
| 4. | a) Give layout of modern steam power plant explain it briefly [5M] | CO 2 |  | Knowledge |
|  | b) Enumerate and explain the steps involved in handling of the coal. [5M] | CO 2 |  | Undeerstand |

| CO’s | PO | PSO |
| --- | --- | --- |
| CO 1 | 1,3,7 | 2 |
| CO 2 | 1,3,7 | 2 |