ADITYA INSTITUTE OF TECHNOLOGY & MANAGEMENT, TEKKALI

AUTONOMOUS

DEPARTMENT OF MECHANICAL ENGINEERING

Class: IV MECH (I Sem)			MID-II		         	             Date: 30-08-2017

Subject: FEM				            2017-18	        	             		Time: 2 Hours

Answer all the following questions.					10x4 = 40M

|  | Course Outcome(s) |
| --- | --- |
| a) The order of element strain energy is _______ (A)3 x 3 (B) 1 x 1 (C) 6 x 6 (D) 6 x 1 | CO2 |
| b) What are the units for element strain displacement matrix B? (A) no units (B) mm (C) /mm (D) N | CO2 |
| c) The half band width NBW for the 1D bar element shown below is ______ (A) 2 (B) 3 (C) 4 (D) 5 | CO2 |
| d) If there is a temperature rise of 300 °C in a 1D bar element with its α= 150 x 10-6/°C, then the initial strain ε0 is ___ (A) 4500 (B) 450 (C) 0.045 (D) 0.0045 | CO2 |
| e) The stiffness values of the constrained nodes are highly increased in ______ method (A) Rayleigh (B) Elimination (C) Penalty (D) Lagrange | CO2 |
| f) The order of stiffness matrix for plane truss element is ______ (A)2 x 2 (B) 4 x 4 (C) 6 x 6 (D) 4 x 1 | CO3 |
| g) The area of CST element is _____ (A) [J] (B) detJ (C) 1/2IdetJI (D) detB | CO2 |
| h) The order of material matrix [D] used in solving CST element problems is _____ (A)3 x 3 (B) 4 x 4 (C) 6 x 6 (D) 6 x 1 | CO3 |
| i) The transformation used between local and global nodal displacements for plane truss element reduced to a 1D bar element is____ (A) [q'] =[K] [Q] (B) [q'] =[L]T [Q] (C) [q'] =[L] [q] (D) [q'] =[A] [q] | CO3 |
| j) In a CST element the order of shape function matrix [N] is _____ (A) 5 x 5 (B) 4x 4 (C) 2 x 2 (D) 3x3 | CO3 |

Part-B

|  |  | Course Outcome | Cognitive Level |
| --- | --- | --- | --- |
| 2 | Consider the bar shown below. An axial load at node 2, P = 3 X 105 N is applied at 30°C to the rod. Then the temperature is raised to 70°C. - Assemble K and F matrices. - Determine nodal displacements and stress in each element, considering that element 1 is made of Aluminum and has E1 = 7 X 1010 N/m2, A1 = 800 mm2, α1= 23 x 10-6/°C and element 2 is made of steel and has E2 = 2 X 1011 N/m2, A2 = 400 mm2 α2= 11.7 x 10-6/°C. [10 M] | CO2 | Application |
| 3 | a) For the pin jointed configuration shown below determine the stiffness values K11, K12, K22 of the global stiffness matrix. [5 M] | CO 3 | Application |
|  | b) For the point P located inside the CST element shown below the shape functions N1 and N2 are 0.15 and 0.25 respectively. Determine coordinates of the point P, and also find the area of the triangle. [5M] | CO 3 | Application |
| 4 | Derive the Jacobian and strain displacement matrix(B) for a CST element used in 2D FE Analysis. [10 M] | CO 3 | Analysis |

CO-PO & PSO Mapping

| CO | CO1 | CO2 | CO3 | CO4 | CO5 |
| --- | --- | --- | --- | --- | --- |
| PO |  | 1,2,3,5 | 1,2,3,5 |  |  |
| PSO |  | 1 | 1 |  |  |

******

ADITYA INSTITUTE OF TECHNOLOGY & MANAGEMENT, TEKKALI

AUTONOMOUS

DEPARTMENT OF MECHANICAL ENGINEERING

Class: IV MECH (I Sem)			MID-II		         	             Date: 30-08-2017

Subject: FEM				            2017-18	        	             		Time: 2 Hours

Answer all the following questions.					10x4 = 40M

|  | Course Outcome(s) |
| --- | --- |
| a) The order of element strain energy is _______ (A)3 x 3 (B) 1 x 1 (C) 6 x 6 (D) 6 x 1 | CO2 |
| b) What are the units for element strain displacement matrix B? (A) no units (B) mm (C) /mm (D) N | CO2 |
| c) The half band width NBW for the 1D bar element shown below is ______ (A) 2 (B) 3 (C) 4 (D) 5 | CO2 |
| d) If there is a temperature rise of 300 °C in a 1D bar element with its α= 150 x 10-6/°C, then the initial strain ε0 is ___ (A) 4500 (B) 450 (C) 0.045 (D) 0.0045 | CO2 |
| e) The stiffness values of the constrained nodes are highly increased in ______ method (A) Rayleigh (B) Elimination (C) Penalty (D) Lagrange | CO2 |
| f) The order of stiffness matrix for plane truss element is ______ (A)2 x 2 (B) 4 x 4 (C) 6 x 6 (D) 4 x 1 | CO3 |
| g) The area of CST element is _____ (A) [J] (B) detJ (C) 1/2IdetJI (D) detB | CO2 |
| h) The order of material matrix [D] used in solving CST element problems is _____ (A)3 x 3 (B) 4 x 4 (C) 6 x 6 (D) 6 x 1 | CO3 |
| i) The transformation used between local and global nodal displacements for plane truss element reduced to a 1D bar element is____ (A) [q'] =[K] [Q] (B) [q'] =[L]T [Q] (C) [q'] =[L] [q] (D) [q'] =[A] [q] | CO3 |
| j) In a CST element the order of shape function matrix [N] is _____ (A) 5 x 5 (B) 4x 4 (C) 2 x 2 (D) 3x3 | CO3 |

Part-B

|  |  | Course Outcome | Cognitive Level |
| --- | --- | --- | --- |
| 2 | Consider the bar shown below. An axial load at node 2, P = 3 X 105 N is applied at 30°C to the rod. Then the temperature is raised to 70°C. - Assemble K and F matrices. - Determine nodal displacements and stress in each element, considering that element 1 is made of Aluminum and has E1 = 7 X 1010 N/m2, A1 = 800 mm2, α1= 23 x 10-6/°C and element 2 is made of steel and has E2 = 2 X 1011 N/m2, A2 = 400 mm2 α2= 11.7 x 10-6/°C. [10 M] | CO2 | Application |
| 3 | a) For the pin jointed configuration shown below determine the stiffness values K11, K12, K22 of the global stiffness matrix. [5 M] | CO 3 | Application |
|  | b) For the point P located inside the CST element shown below the shape functions N1 and N2 are 0.15 and 0.25 respectively. Determine coordinates of the point P, and also find the area of the triangle. [5M] | CO 3 | Application |
| 4 | Derive the Jacobian and strain displacement matrix(B) for a CST element used in 2D FE Analysis. [10 M] | CO 3 | Analysis |

CO-PO & PSO Mapping

| CO | CO1 | CO2 | CO3 | CO4 | CO5 |
| --- | --- | --- | --- | --- | --- |
| PO |  | 1,2,3,5 | 1,2,3,5 |  |  |
| PSO |  | 1 | 1 |  |  |

******