ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AUTONOMOUS)

MID – I                                 Date: 21-12-2017

Branch: III ME_A&B	     Year / Semester: III –II (2017-18)                        Time: 2 hrs                     Subject: Heat Transfer	                                                                       	       Marks: 40

Answer all Questions and carry equal marks.

(Use of Heat Transfer data book is permitted.)

PART- A                                     10x1 = 10 M

|  | QUESTIONS | Course Outcome |
| --- | --- | --- |
| (1).i | In a case of one dimensional heat conduction in a medium with constant properties, T is the temperature at position x, at time t. Then ∂T∂t is proportional to: (a) Tx (b) ∂T∂x (c) ∂T∂x.∂t (d) ∂2T∂x2 | CO1 |
| ii | Thermal diffusivity of a substance is: (a) Inversely proportional to thermal conductivity (b) Directly proportional to thermal conductivity (c) Directly proportional to the square of thermal conductivity (d) Inversely proportional to the square of thermal conductivity | CO1 |
| iii | Overall heat transfer coefficient is associated with (a) Conduction and radiation (b) Convection and conduction (c) Radiation and convection (d) None | CO1 |
| iv | Temperature variation in a plain wall without internal heat generation is (a) linear (b) Parabolic (c) logarithmic (d) None | CO1 |
| v | Temperature variation in a pipe without internal heat generation is (a) linear (b) Parabolic (c) logarithmic (d) None | CO1 |
| vi | In which one of the following materials, is the heat energy propagation minimum due to conduction heat transfer? (a) Lead (b) Copper (c) Water (d) Air | CO1 |
| vii | Heat transfer from higher temperature to lower temperature takes place according to a) Fourier’s law b) First law of thermodynamics c) Second law of thermodynamics d) Zeroth law of thermodynamics | CO1 |
| viii | Heat is transfer by all three modes of transfer , viz ,conduction, convection and radiation in (a) electric heater (b)steam condenser (c)melting of ice (d)boiler | CO1 |
| ix | In case of heat conduction through plane wall, which one of the following is not a correct assumption? a) Steady state b) Three dimensional heat flow c) Volumetric heat generation must be constant d) K must be constant | CO2 |
| x | The temperature drop in a plane wall with uniformly distributed heat generation can be decreased by reducing a) Wall thickness b) Heat generation rate c) Thermal conductivity d) Surface area | CO2 |

PART – B

| Q. No. | Question | Course outcome (s) | Cognitive Level |
| --- | --- | --- | --- |
| 2. | (a)Explain clearly the mechanism of conduction, convection and radiation. [5M] | CO1 | Knowledge |
|  | (b) A composite slab consists of 250 mm fire clay brick (k=1.09 W/m-K) inside, 100 mm fired earth brick (0.26 W/m-K) and outer layer of common brick (0.6 W/m-K) of thickness 50 mm. If inside surface is at 12000C and outside surface is at 1000C, find: (i) heat flux. (ii) the temperature of the junctions [5M] | CO1 | Application |
| 3. | Derive the general heat conduction equation for cylindrical coordinate system and simplify to one-dimensional steady state without heat generation and constant thermal conductivity in radial direction. [10M] | CO1 | Analysis |
| 4. | Derive expressions for temperature distribution during steady sate heat conduction in a plane wall with internal heat generation. Assume the temperature on the two faces of the wall will be the same. [10M] | CO2 | Analysis |

| CO | PO | PSO |
| --- | --- | --- |
| 1 | 1,2,3 | 2 |
| 2 | 1,2,3 | 2 |

******

ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AUTONOMOUS)

MID – I                                  Date: 21-12-2017

Branch: III ME_A&B	     Year / Semester: III –II (2017-18)                         Time: 2 hrs                     Subject: Heat Transfer	                                                                       	                                Marks: 40

Answer all Questions and carry equal marks.

(Use of Heat Transfer data book is permitted.)

PART- A                                     10x1 = 10 M

|  | QUESTIONS | Course Outcome |
| --- | --- | --- |
| (1).i | In a case of one dimensional heat conduction in a medium with constant properties, T is the temperature at position x, at time t. Then ∂T∂t is proportional to: (a) Tx (b) ∂T∂x (c) ∂T∂x.∂t (d) ∂2T∂x2 | CO1 |
| ii | Thermal diffusivity of a substance is: (a) Inversely proportional to thermal conductivity (b) Directly proportional to thermal conductivity (c) Directly proportional to the square of thermal conductivity (d) Inversely proportional to the square of thermal conductivity | CO1 |
| iii | Overall heat transfer coefficient is associated with (a) Conduction and radiation (b) Convection and conduction (c) Radiation and convection (d) None | CO1 |
| iv | Temperature variation in a plain wall without internal heat generation is (a) linear (b) Parabolic (c) logarithmic (d) None | CO1 |
| v | Temperature variation in a pipe without internal heat generation is (a) linear (b) Parabolic (c) logarithmic (d) None | CO1 |
| vi | In which one of the following materials, is the heat energy propagation minimum due to conduction heat transfer? (a) Lead (b) Copper (c) Water (d) Air | CO1 |
| vii | Heat transfer from higher temperature to lower temperature takes place according to a) Fourier’s law b) First law of thermodynamics c) Second law of thermodynamics d) Zeroth law of thermodynamics | CO1 |
| viii | When the thickness of insulation on a pipe exceeds the critical value (a) heat flow rate decrease (b)heat flow rate increase (c) heat flow rate remains constant (d)none of the above | CO1 |
| ix | In case of heat conduction through plane wall, which one of the following is not a correct assumption? a) Steady state b) Three dimensional heat flow c) Volumetric heat generation must be constant d) K must be constant | CO2 |
| x | The temperature drop in a plane wall with uniformly distributed heat generation can be decreased by reducing a) Wall thickness b) Heat generation rate c) Thermal conductivity d) Surface area | CO2 |

PART – B

| Q. No. | Question | Course outcome (s) | Cognitive Level |
| --- | --- | --- | --- |
| 2. | (a)Explain clearly the mechanism of conduction, convection and radiation. [5M] | CO1 | Knowledge |
|  | (b) A composite slab consists of 250 mm fire clay brick (k=1.09 W/m-K) inside, 100 mm fired earth brick (0.26 W/m-K) and outer layer of common brick (0.6 W/m-K) of thickness 50 mm. If inside surface is at 12000C and outside surface is at 1000C, find: (i) heat flux. (ii) the temperature of the junctions [5M] | CO1 | Application |
| 3. | Derive the general heat conduction equation for cylindrical coordinate system and simplify to one-dimensional steady state without heat generation and constant thermal conductivity in radial direction. [10M] | CO1 | Analysis |
| 4. | Derive expressions for temperature distribution during steady sate heat conduction in a plane wall with internal heat generation. Assume the temperature on the two faces of the wall will be the same. [10M] | CO2 | Analysis |

| CO | PO | PSO |
| --- | --- | --- |
| 1 | 1,2,3 | 2 |
| 2 | 1,2,3 | 2 |

******