ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

DEPARTMENT OF MECHANICAL ENGINEERING

MID: I

CLASS: 3rd ME        SEC: A&B						SEMESTER: II

SUB: Metrology				SET-1			           DATE: 19-12-2017

Answer All Questions

|  | CO |
| --- | --- |
| 1. Tolerances are specified: (a) To Obtain desired fits (b) Because it is not possible to manufacture a size exactly. (c) To Obtain high accuracy (d) To have proper inspection. | CO1 |
| 2. In hole basis system, basic size is equal to a. maximum hole size b. minimum hole size c. upper deviation which is zero d. none of the above | CO1 |
| 3. Expressing a dimension as 20.3 ± 0.02 mm is the case of: (a) Limiting dimension (b) Bilateral dimension (c) Unilateral dimension (d) All of the above | CO1 |
| 4. A negative allowance will always result in a _____ fit. (a) Clearance (b) Interference (c)Transition (d) Any of the above | CO1 |
| 5 Fitting of rim on a locomotive wheel is done by (a)Keying fit (b) Driving fit (c)Force fit (d)Any of the above | CO1 |
| 6. The fit on a hole-shaft system is specified as H11 - c11. The type of fit is: a) close running fit b) force fit (interference fit) c) push fit (transition fit) d) loose running fit | CO1 |
| 7. The following is a line standard measurement (a) Measuring tape (b) Slip gauges (c) Micrometer (d) End bars | CO2 |
| 8. Engineer's steel rule is a precision measuring instrument and must be treated as such and kept in a a) polished condition b) fine condition c) corrosion condition d)none of the above | CO2 |
| 9.The degree of closeness of the measured value of a certain quantity with its true value is known as (a) Accuracy (b) Precision (c) Standard (d) Sensitivity | CO2 |
| 10. Least count of Micrometer Screw Gauge is a) 1mm b) 0.001mm c)0.1mm d)0.01mm | CO2 |

PART-B

|  |  | Course outcome (s) | Cognitive Level |
| --- | --- | --- | --- |
| 2 | a) Define interchangeability and discuss its importance. (5M) b) What are the types of fit and discuss them with sketch.(5M) | CO 1 | Knowledge, Comprehension |
| 3 | A hole and shaft have a basic size of 25 mm, and are to have a clearance fit with maximum clearance of 0.02mm and a minimum clearance of 0.01mm. The hole tolerance is to be 1.5 times the shaft tolerance. Determine the limits for both hole and shaft (a) using a hole basis system (b) using a shaft basis system. (10M) | CO 1 | Application |
| 4 | a) How the measuring instruments / methods are classified? (5M) b)Explain the slip gauges and also the care to be taken while handling them. (5M) | CO 2 | Comprehension, |

CO-PO MAPPING

| CO | CO1 | CO2 |
| --- | --- | --- |
| PO | 1,2,13 | 1,2,13 |
| PSO | 3 | 2 |

******

ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

DEPARTMENT OF MECHANICAL ENGINEERING

MID: I

CLASS: 3rd ME        SEC: A&B						SEMESTER: II

SUB: Metrology				SET-1			           DATE: 19-12-2017

Answer All Questions

|  | CO |
| --- | --- |
| 1. Tolerances are specified: (a) To Obtain desired fits (b) Because it is not possible to manufacture a size exactly. (c) To Obtain high accuracy (d) To have proper inspection. | CO1 |
| 2. In hole basis system, basic size is equal to a. maximum hole size b. minimum hole size c. upper deviation which is zero d. none of the above | CO1 |
| 3. Expressing a dimension as 20.3 ± 0.02 mm is the case of: (a) Limiting dimension (b) Bilateral dimension (c) Unilateral dimension (d) All of the above | CO1 |
| 4. A negative allowance will always result in a _____ fit. (a) Clearance (b) Interference (c)Transition (d) Any of the above | CO1 |
| 5 Fitting of rim on a locomotive wheel is done by (a)Keying fit (b) Driving fit (c)Force fit (d)Any of the above | CO1 |
| 6. The fit on a hole-shaft system is specified as H11 - c11. The type of fit is: a) close running fit b) force fit (interference fit) c) push fit (transition fit) d) loose running fit | CO1 |
| 7. The following is a line standard measurement (a) Measuring tape (b) Slip gauges (c) Micrometer (d) End bars | CO2 |
| 8. Engineer's steel rule is a precision measuring instrument and must be treated as such and kept in a a) polished condition b) fine condition c) corrosion condition d)none of the above | CO2 |
| 9.The degree of closeness of the measured value of a certain quantity with its true value is known as (a) Accuracy (b) Precision (c) Standard (d) Sensitivity | CO2 |
| 10. Least count of Micrometer Screw Gauge is a) 1mm b) 0.001mm c)0.1mm d)0.01mm | CO2 |

PART-B

|  |  | Course outcome (s) | Cognitive Level |
| --- | --- | --- | --- |
| 2 | a) Define interchangeability and discuss its importance. (5M) b) What are the types of fit and discuss them with sketch.(5M) | CO 1 | Knowledge, Comprehension |
| 3 | A hole and shaft have a basic size of 25 mm, and are to have a clearance fit with maximum clearance of 0.02mm and a minimum clearance of 0.01mm. The hole tolerance is to be 1.5 times the shaft tolerance. Determine the limits for both hole and shaft (a) using a hole basis system (b) using a shaft basis system. (10M) | CO 1 | Application |
| 4 | a) How the measuring instruments / methods are classified? (5M) b)Explain the slip gauges and also the care to be taken while handling them. (5M) | CO 2 | Comprehension, |

CO-PO MAPPING

| CO | CO1 | CO2 |
| --- | --- | --- |
| PO | 1,2,13 | 1,2,13 |
| PSO | 3 | 2 |

******