ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AUTONOMOUS)

MID – II                                        Date: 08-02-2018

Branch: III ME_A&B	     Year / Semester: III –II (2017-18)           Time: 2 hrs

Subject: Heat Transfer	                                                                       Marks: 40

Answer all Questions and carry equal marks.

(Use of Heat Transfer data book is permitted.)

PART- A                                     10x1 = 10 M

| S. No. | QUESTIONS | Course Outcome |
| --- | --- | --- |
| 1.(i) | The value of Biot number is very small (less than 0.01) when (a) The convective resistance of the fluid is negligible (b) The conductive resistance of the fluid is negligible (c) The conductive resistance of the solid is negligible (d) None of these | CO2 |
| ii | (iii) The sensitivity of a thermocouple is the time required for a thermocouple to reach------ %of the initial temperature difference (a) 0.3 (b)0.368 (c) 0.5 (d) 0.638 | CO2 |
| iii | On heat transfer surface, fins are provided (a) To increase temperature gradient so as to enhance heat transfer (b) To increase turbulence in flow for enhancing heat transfer (c) To increase surface area to promote the rate of heat transfer (d) To decrease the pressure drop of the fluid | CO2 |
| iv | Addition of fin to the surface increases the heat transfer if hA / KP is (a) Equal to one (b) Greater than one (c) Less than one (d) Greater than one but less than two | CO2 |
| v | The conditions which are important to study the analysis of a heat transfer process under study state are (a)geometric conditions (b)physical conditions (c)boundary conditions (d) all of the above | CO3 |
| vi | (vii)The ratio of energy transferred by convection to that by conduction is called (a) Stanton number (b) Nusselt number (c) Biot number (d) Peclet number | CO3 |
| vii | Which one of the following numbers represents the ratio of kinematic viscosity to the thermal diffusivity? (a) Grashoff number (b) Prandtl number (c) Mach number (d) Nusselt number | CO3 |
| viii | Viscous effects of a fluid (a)is found in the turbulent core (b)is confined outside the boundary layer (c)is confined in the boundary layer (d)is confined in the potential flow | CO3 |
| ix | (vii)The efficiency of a pin fin with insulated tip is (CO2) (a) tan h mL/(ha/kp)0.5 ( b) tan h mL/mL (c) mL/tan h mL (d) none | CO2 |
| x | Heisler charts are used to determine transient heat flow rate and temperature distribution when: (a) Solids possess infinitely large thermal conductivity (b) Internal conduction resistance is small and convective resistance is large (c) Internal conduction resistance is large and the convective resistance is small (d) Both conduction and convention resistance are almost of equal significance | CO2 |

PART – B

| Q. No. | Questions | Course outcome (s) | Cognitive Level |
| --- | --- | --- | --- |
| 2. | (a) What is meant by lumped capacity? What are the assumptions involved and under what conditions this method can be applied? [5M] | CO2 | Knowledge |
|  | (b)A stainless steel rod of outer diameter 1 cm originally at a temperature of 320 ºC is suddenly immersed in a liquid at 120 ºC for which the convective heat transfer coefficient is 100 W/m2-K. Determine the time required for the rod to reach a temperature of 200 ºC? [5M] | CO2 | Analysis |
| 3. | Derive the expression for heat transfer for the case of a straight rectangular fin of uniform cross section when the tip of the fin is end insulated. [10M] | CO3 | Knowledge &application |
| 4 | State the Buckingham’s π Theorem. Explain the various parameters used in forced convection. Using dimensional analysis obtain an expression for Nusselt number in terms of Reynolds and Prandtl numbers? [10M] | CO3 | Analysis |

| CO | PO | PSO |
| --- | --- | --- |
| 2 | 1,2,3,4 | 2 |
| 3 | 1,2,3,4,5 | 2 |

******

ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AUTONOMOUS)

MID – II                                        Date: 08-02-2018

Branch: III ME_A&B	     Year / Semester: III –II (2017-18)           Time: 2 hrs

Subject: Heat Transfer	                                                                       Marks: 40

Answer all Questions and carry equal marks.

(Use of Heat Transfer data book is permitted.)

PART- A                                     10x1 = 10 M

| S. No. | QUESTIONS | Course Outcome |
| --- | --- | --- |
| 1.(i) | The value of Biot number is very small (less than 0.01) when (a) The convective resistance of the fluid is negligible (b) The conductive resistance of the fluid is negligible (c) The conductive resistance of the solid is negligible (d) None of these | CO2 |
| ii | (iii) The sensitivity of a thermocouple is the time required for a thermocouple to reach------ %of the initial temperature difference (a) 0.3 (b)0.368 (c) 0.5 (d) 0.638 | CO2 |
| iii | On heat transfer surface, fins are provided (a) To increase temperature gradient so as to enhance heat transfer (b) To increase turbulence in flow for enhancing heat transfer (c) To increase surface area to promote the rate of heat transfer (d) To decrease the pressure drop of the fluid | CO2 |
| iv | Addition of fin to the surface increases the heat transfer if hA / KP is (a) Equal to one (b) Greater than one (c) Less than one (d) Greater than one but less than two | CO2 |
| v | The conditions which are important to study the analysis of a heat transfer process under study state are (a)geometric conditions (b)physical conditions (c)boundary conditions (d) all of the above | CO3 |
| vi | (vii)The ratio of energy transferred by convection to that by conduction is called (a) Stanton number (b) Nusselt number (c) Biot number (d) Peclet number | CO3 |
| vii | Which one of the following numbers represents the ratio of kinematic viscosity to the thermal diffusivity? (a) Grashoff number (b) Prandtl number (c) Mach number (d) Nusselt number | CO3 |
| viii | Viscous effects of a fluid (a)is found in the turbulent core (b)is confined outside the boundary layer (c)is confined in the boundary layer (d)is confined in the potential flow | CO3 |
| ix | (vii)The efficiency of a pin fin with insulated tip is (CO2) (a) tan h mL/(ha/kp)0.5 ( b) tan h mL/mL (c) mL/tan h mL (d) none | CO2 |
| x | Heisler charts are used to determine transient heat flow rate and temperature distribution when: (a) Solids possess infinitely large thermal conductivity (b) Internal conduction resistance is small and convective resistance is large (c) Internal conduction resistance is large and the convective resistance is small (d) Both conduction and convention resistance are almost of equal significance | CO2 |

PART – B

| Q. No. | Questions | Course outcome (s) | Cognitive Level |
| --- | --- | --- | --- |
| 2. | (a) What is meant by lumped capacity? What are the assumptions involved and under what conditions this method can be applied? [5M] | CO2 | Knowledge |
|  | (b)A stainless steel rod of outer diameter 1 cm originally at a temperature of 320 ºC is suddenly immersed in a liquid at 120 ºC for which the convective heat transfer coefficient is 100 W/m2-K. Determine the time required for the rod to reach a temperature of 200 ºC? [5M] | CO2 | Analysis |
| 3. | Derive the expression for heat transfer for the case of a straight rectangular fin of uniform cross section when the tip of the fin is end insulated. [10M] | CO3 | Knowledge &application |
| 4 | State the Buckingham’s π Theorem. Explain the various parameters used in forced convection. Using dimensional analysis obtain an expression for Nusselt number in terms of Reynolds and Prandtl numbers? [10M] | CO3 | Analysis |

| CO | PO | PSO |
| --- | --- | --- |
| 2 | 1,2,3,4 | 2 |
| 3 | 1,2,3,4,5 | 2 |

******