ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AUTONOMOUS)

MID – III                                              Date: 23-03-2018

Branch: III ME_A&B	     Year / Semester: III –II (2017-18)                   Time: 2 hrs

Subject: Heat Transfer	                                                                                Marks: 40

Answer all Questions and carry equal marks.

(Use of Heat Transfer data book is permitted.)

PART- A                                     10x1 = 10 M

|  | QUESTIONS | Course Outcome |
| --- | --- | --- |
| 1.i | The ratio of buoyancy force to viscous force acting on a fluid is called (a) Reynolds number (b) Grashoff's number (c) Nusselt number (d) Prandtl number | CO4 |
| ii | Free convection flow depends on all of the following EXCEPT (a) Density (b) Coefficient of viscosity (c) Gravitational force (d) Velocity | CO4 |
| iii | The 'NTU' in a heat exchanger is given by which one of the following? (CO4) (a)UACmin (b) UACmax (c) UAE (d) UACmax | CO4 |
| iv | Compared to parallel flow heat exchanger, the LMTD of a counter flow heat exchanger is (a) more (b) less (c) same (d) none of the above | CO4 |
| v | In a heat exchanger with one fluid evaporating or condensing the surface area required is least in (a) Parallel flow (b) Counter flow (c) Cross flow (d) Same in all of the above | CO4 |
| vi | When α is absorbtivity, ρ is reflectivity and τ is transmisivity, then for diathermanous body, which of the following relation is valid? (a) α = 1, ρ = 0, τ = 0 (b) α = 0, ρ = 1, τ = 0 (c) α = 0, ρ = 0, τ = 1 (d) α + ρ = 1, τ = 0 | CO5 |
| vii | What is the basic equation of thermal radiation from which all other equations of radiation can be derived? (a) Stefan-Boltzmann equation (b) Planck’s equation (c) Wien’s equation (d) Rayleigh-Jeans formula | CO5 |
| viii | Ice is very close to a (a) Gray body (b)black body (c) white body (d) specular body | CO5 |
| ix | When a body is in thermal equilibrium with its surroundings, the emissivity of its surface is equal to its absorpvity this is called (a) Kirchhoff’s law (b) Wien’s law (c) Planck’s law (d) None of the above | CO5 |
| x | In radiative heat transfer, a gray surface is one (a) Which appears gray to the eye (b) Whose emissivity is independent of wavelength (c) Which has reflectivity equal to zero (d) Which appears equally bright from all directions. | CO5 |

PART – B

| Q. No | Questions | Course outcome(s) | Cognitive Level |
| --- | --- | --- | --- |
| 2. | (a) Give the classification of heat exchangers with the help of simple sketches? [3M] | CO4 | Knowledge |
|  | (b) In a certain double pipe heat exchanger hot water flows at a rate of 5000 kg/h and gets cooled from 95oC to 65oC. At the same time 50000 kg/h of cooling water at 30oC enters the heat exchanger. The flow conditions are such that overall heat transfer coefficient remains constant at 2270 W/m2 K. Determine the heat transfer area required and the effectiveness, assuming two streams are in parallel flow. Assume for the both the streams cp = 4.2 kJ/kg K. [7M] | CO4 | Application |
| 3. | Derive the Nusselt theory of laminar flow film condensation on a vertical plate. [10M] | CO4 | Analysis |
| 4 | (a) Define the terms: i) Irradiation ii) Radiosity iii) Emissivity [3M] | CO5 | Knowledge |
|  | (b)A pipe carrying steam having outside diameter 20 cm runs in a large room and is exposed air at a temperature of 300C.The pipe surface temperature is 4000C.Calculate the heat lossto the surroundings per metre length of the pipe due to thermal radiation .The emissivity of the pipr surface is 0.8.What would be the loss of heat due to radiation if the pipe is enclosed in a 40 cm diameter brick conduict of emissivity is 0.91 . [7M] | CO5 | Application |

| CO | PO | PSO |
| --- | --- | --- |
| 3 | 1,2,3,4 | 2 |

ADITYA INSTITUTE OF TECHNOLOGY AND MANAGEMENT, TEKKALI

(AUTONOMOUS)

MID – III                                              Date: 23-03-2018

Branch: III ME_A&B	     Year / Semester: III –II (2017-18)                   Time: 2 hrs

Subject: Heat Transfer	                                                                                Marks: 40

Answer all Questions and carry equal marks.

(Use of Heat Transfer data book is permitted.)

PART- A                                     10x1 = 10 M

|  | QUESTIONS | Course Outcome |
| --- | --- | --- |
| 1.i | The ratio of buoyancy force to viscous force acting on a fluid is called (a) Reynolds number (b) Grashoff's number (c) Nusselt number (d) Prandtl number | CO4 |
| ii | Free convection flow depends on all of the following EXCEPT (a) Density (b) Coefficient of viscosity (c) Gravitational force (d) Velocity | CO4 |
| iii | The 'NTU' in a heat exchanger is given by which one of the following? (CO4) (a)UACmin (b) UACmax (c) UAE (d) UACmax | CO4 |
| iv | Compared to parallel flow heat exchanger, the LMTD of a counter flow heat exchanger is (a) more (b) less (c) same (d) none of the above | CO4 |
| v | In a heat exchanger with one fluid evaporating or condensing the surface area required is least in (a) Parallel flow (b) Counter flow (c) Cross flow (d) Same in all of the above | CO4 |
| vi | When α is absorbtivity, ρ is reflectivity and τ is transmisivity, then for diathermanous body, which of the following relation is valid? (a) α = 1, ρ = 0, τ = 0 (b) α = 0, ρ = 1, τ = 0 (c) α = 0, ρ = 0, τ = 1 (d) α + ρ = 1, τ = 0 | CO5 |
| vii | What is the basic equation of thermal radiation from which all other equations of radiation can be derived? (a) Stefan-Boltzmann equation (b) Planck’s equation (c) Wien’s equation (d) Rayleigh-Jeans formula | CO5 |
| viii | Ice is very close to a (a) Gray body (b)black body (c) white body (d) specular body | CO5 |
| ix | When a body is in thermal equilibrium with its surroundings, the emissivity of its surface is equal to its absorpvity this is called (a) Kirchhoff’s law (b) Wien’s law (c) Planck’s law (d) None of the above | CO5 |
| x | In radiative heat transfer, a gray surface is one (a) Which appears gray to the eye (b) Whose emissivity is independent of wavelength (c) Which has reflectivity equal to zero (d) Which appears equally bright from all directions. | CO5 |

PART – B

| Q. No | Questions | Course outcome(s) | Cognitive Level |
| --- | --- | --- | --- |
| 2. | (a) Give the classification of heat exchangers with the help of simple sketches? [3M] | CO4 | Knowledge |
|  | (b) In a certain double pipe heat exchanger hot water flows at a rate of 5000 kg/h and gets cooled from 95oC to 65oC. At the same time 50000 kg/h of cooling water at 30oC enters the heat exchanger. The flow conditions are such that overall heat transfer coefficient remains constant at 2270 W/m2 K. Determine the heat transfer area required and the effectiveness, assuming two streams are in parallel flow. Assume for the both the streams cp = 4.2 kJ/kg K. [7M] | CO4 | Application |
| 3. | Derive the Nusselt theory of laminar flow film condensation on a vertical plate. [10M] | CO4 | Analysis |
| 4 | (a) Define the terms: i) Irradiation ii) Radiosity iii) Emissivity [3M] | CO5 | Knowledge |
|  | (b)A pipe carrying steam having outside diameter 20 cm runs in a large room and is exposed air at a temperature of 300C.The pipe surface temperature is 4000C.Calculate the heat lossto the surroundings per metre length of the pipe due to thermal radiation .The emissivity of the pipr surface is 0.8.What would be the loss of heat due to radiation if the pipe is enclosed in a 40 cm diameter brick conduict of emissivity is 0.91 . [7M] | CO5 | Application |

| CO | PO | PSO |
| --- | --- | --- |
| 3 | 1,2,3,4 | 2 |